Task 3: Communication Data, Signals, Events and Protocols

Team 5, Autonomous Systems A Lab

Data, signals and events the leader and follower truck communicate in each scenario: what it is, sender, receiver, and timing. Each scenario's protocol follows, and all three were modelled as timed automata in UPPAAL and verified.

# Scenario 1: Normal Platooning

Description: Leader keeps sending speed and position while the follower stays in lane using the camera and keeps a safe distance. Nothing has failed.

| Name | Type | Sender | Receiver | Meaning | Timing constraint |
| --- | --- | --- | --- | --- | --- |
| leader_speed_position | Data | Leader Truck | Follower Truck | The leader keeps sending its current speed and position so the follower can match it. | Sent and received within 100ms (TR3) |
| camera_frame | Data | Camera | Lane Detection Model | Image frame taken at 30 FPS for lane detection. | Captured and processed within 33ms (TR1) |
| lane_position | Data | Lane Detection Model | Follower Truck | The detected lane markings used for steering. | Within one frame cycle, 33ms (TR2) |
| steering_correction | Signal | Follower Truck | Steering Controller | Command to keep the truck inside its lane. | Applied within 33ms (TR2) |
| following_distance | Data | Dist/Speed Controller | Follower Truck | The gap to the leader, checked against the 5m minimum. | Continuous (FR3) |
| distance_below_min | Event | Dist/Speed Controller | Follower Truck | Fires when the gap drops below 5m so the truck reduces speed. | Immediate (SR1) |
| lane_lost_alert | Event | Lane Detection Model | Leader Truck | Raised when lane markings are missing for more than 3 frames, so the truck slows and alerts. | After 3 frames (SR2) |
| speed_command | Signal | Dist/Speed Controller | Throttle Actuator | Adjusts speed to match the leader without going faster than it. | Speed sync within 100ms (TR3, SR3) |

Protocol: Leader sends speed and position every 100ms (TR3). Follower processes each camera frame within 33ms (TR1) and applies a steering correction in the same frame (TR2). If distance drops below 5m the truck reduces speed (SR1), otherwise it matches the leader's speed without exceeding it (FR4, SR3). If lane markings are lost for more than 3 frames the truck slows and alerts (SR2).

# Scenario 2: Communication Failure

Description: Wireless link drops and the follower stops receiving beacons. It must detect this within 500ms and switch to a fallback mode, slowing down safely until the link returns or it stops.

| Name | Type | Sender | Receiver | Meaning | Timing constraint |
| --- | --- | --- | --- | --- | --- |
| leader_speed_position | Data | Leader Truck | Follower Truck | The normal speed and position beacon. When it stops arriving, that means the link is lost. | Period 100ms or less |
| timeout_reset | Event | Follower Truck | Timeout Timer | Resets the timeout timer every time a beacon comes in (FR1). | On every received signal |
| signal_timeout | Event | Timeout Timer | Follower Truck | Fires when no beacon has arrived for 500ms, which counts as a communication failure. | Within 500ms of last signal (TR1, FR2) |
| switch_to_fallback | Signal | Follower Truck | Dist/Speed Controller | Tells the truck to leave platooning mode and go into fallback mode. | Within 100ms of detection (TR2) |
| decelerate_command | Signal | Follower Truck | Throttle Actuator | Slows the truck down smoothly toward a safe speed or a stop, no hard braking. | Starts within 600ms of last signal (TR3, SR2) |
| camera_lane_position | Data | Camera / Lane Detection Model | Follower Truck | Lane keeping keeps running off the camera during fallback. | Frame processing 33ms or less (FR5, SR3) |
| link_restored | Event | Wireless Link | Follower Truck | The beacon comes back before 10s and the truck goes back to platooning. | Before 10s |
| full_stop_command | Signal | Follower Truck | Throttle Actuator | If the link is still gone after 10s, bring the truck to a full stop. | At 10s (SR4) |

Protocol: Follower resets its timeout timer on every beacon (FR1). No beacon for 500ms triggers a failure (TR1, FR2). Follower switches to fallback within 100ms (TR2) and begins decelerating within 600ms (TR3, SR2), while the camera keeps lane-keeping active (FR5, SR3). If the link returns before 10s the truck resumes platooning; otherwise it comes to a full stop at 10s (SR4).

# Scenario 3: Emergency Braking

Description: An obstacle appears ahead of the leader, which brakes hard and signals the follower. The follower must brake within 200ms, or fall back to spotting the leader's deceleration via camera.

| Name | Type | Sender | Receiver | Meaning | Timing constraint |
| --- | --- | --- | --- | --- | --- |
| emergency_brake_signal | Signal | Leader Truck | Follower Truck | A high priority message sent the moment the leader brakes, telling the follower to brake too. | Received and acted on within 200ms (TR1) |
| obstacle_detected | Event | Leader Truck | Leader Truck | Fires when an obstacle suddenly shows up ahead of the leader. | Immediate (FR1) |
| brake_signal_received | Event | Wireless Link | Follower Truck | The follower gets the brake signal in time (Path A). | Within 200ms (TR1) |
| brake_signal_missed | Event | Follower Truck | Follower Truck | No brake signal arrived in time, so the truck falls back to the camera (Path B). | After signal timeout |
| camera_deceleration_detected | Event | Camera / Lane Detection Model | Follower Truck | The camera sees the leader slowing down, for example its brake lights or the gap closing. | Frame 33ms or less (TR3), braking within 300ms (TR2) |
| precautionary_stop | Event | Follower Truck | Braking System | Both paths failed within 400ms, so the truck does a safe controlled stop (Path C). | If no braking by 400ms (SR2) |
| apply_braking_command | Signal | Follower Truck | Braking System | Applies controlled braking force based on how hard the leader is braking. | Smooth and controlled (FR5, SR3) |

Protocol: Two independent paths so neither the link nor the camera is a single point of failure (NFR2, SR1). Path A: signal received, brake within 200ms (TR1). Path B: signal missed, camera detects deceleration and triggers braking within 300ms (TR2). Path C: if neither path brakes within 400ms, precautionary controlled stop (SR2). Braking stays smooth and in-lane throughout (FR5, SR3, SR4).

# Formal Verification in UPPAAL

Each scenario was built as a network of timed automata in UPPAAL and checked: no deadlock, all key states reachable (fallback, safe stop, speed matching), and all timing bounds hold (500ms failure detection, 100ms mode switch, 200ms signal braking, 300-400ms camera braking, 33ms frame processing).